RTOS Mini Project

Smart Home System using RTOS Concepts (Simulated Sensors)

| S.No | Name | Registeration Number | Branch |
| --- | --- | --- | --- |
| 1 | Adithya Rao Kalathur | 251100610012 | CSE |
| 2 | Ashirvad B M | 251100610024 | CSE |
| 3 | Harish Kumar D | 251100610006 | CSE |

Objective

To simulate a smart home environment controlled by an embedded RTOS, demonstrating key RTOS features such as:

- Periodic Tasks

- Semaphores (with timeouts)

- Message Boxes (with timeouts)

- Mutex with Priority Inheritance

- Priority Ceiling Protocols (ICPP and OCPP simulations)

- Event Flags and Task Communication

Platform

- Microcontroller: LPC2378 (ARM7TDMI-S Core)

- RTOS: Keil RTX (CMSIS-RTOS style, via <RTL.h>)

- IDE: Keil µVision

- Peripherals Simulated:

  - Temperature Sensor

  - Light Sensor

  - Motion Detector

  - LCD Display

  - LEDs for fan/light indicators

System Description

The project emulates a Smart Home Controller that monitors and reacts to:

- Temperature: Controls fan speed (via LEDs).

- Light Level: Adjusts room lighting (via LEDs).

- Motion: Overrides light control temporarily for safety/visibility.

- Clock: Blinks periodically as a system heartbeat.

- Logger: Displays recent system actions.

- Emergency Monitor: Alerts in case of overheating.

Each module is implemented as a separate RTOS task, coordinated through synchronization primitives.

Hardware / Software Components Used

| Component | Type | Purpose |
| --- | --- | --- |
| LPC2378 MCU | Hardware | Main processing core with GPIO for LED/LCD |
| LEDs (P2.0–P2.7) | Hardware | Indicate system states (fan, light, clock) |
| LCD (16x2) | Hardware | Display system information and logs |
| Keil RTX (RTOS) | Software | Manages multitasking and synchronization |
| Semaphores | Software | Protect shared sensor data |
| Mutexes | Software | Protect LCD and ceiling manager |
| Message Box | Software | Queue for log messages |
| Timers/Delays | Software | Used to simulate real-time behavior |

System Architecture

Task Summary

| Task Name | Function | Priority | Key RTOS Features Used |
| --- | --- | --- | --- |
| temp_task | Simulates temperature & controls fan LEDs | 3 | Semaphore, ICPP, MessageBox |
| light_task | Simulates light level & controls lighting LEDs | 4 | Semaphore, OCPP, Event Flags |
| motion_task | Simulates motion detection events | 2 | Event flags |
| display_task | Displays live readings on LCD | 5 | Semaphore, Mutex |
| logger_task | Displays recent log on LCD | 6 | MessageBox, Mutex |
| emergency_task | Detects overheating & flashes LEDs | 1 | Mutex |
| clock_task | LED heartbeat for system activity | 7 | Delay (Periodic task) |

RTOS Concepts Demonstrated

Periodic Tasks

All tasks in this demo are periodic.
They perform actions, delay for a defined period, and repeat.

| Task | Mechanism |
| --- | --- |
| temp_task() | os_dly_wait(200); — every 200 ticks |
| light_task() | os_dly_wait(50); — faster updates |
| motion_task() | os_dly_wait(800); — detects motion periodically |
| clock_task() | os_dly_wait(5/95); — blinking heartbeat |
| emergency_task() | os_dly_wait(50); — checks overheating regularly |

Semaphore with Timeout

Used for protecting shared sensor data between multiple tasks.

if (os_sem_wait(&sem_sensors, 50) == OS_R_OK) {

sensor_temp = temp;

os_sem_send(&sem_sensors);

}

- Used In: temp_task(), light_task(), display_task()

- Timeout: 50 ticks (prevents deadlock)

- Purpose: Avoids race conditions while updating sensor_temp and sensor_light.

Message Box with Timeout

Used to log system activities asynchronously.

os_mbx_send(&msgbox, (void*)log_pool[idx], 40);

os_mbx_wait(&msgbox, &msg, 120);

- Used In:

  - temp_task() → sends temperature logs

  - light_task() → sends light logs

  - logger_task() → waits and displays logs

- Timeouts:

  - Send: 40 ticks

  - Receive: 120 ticks

This allows producer-consumer synchronization between tasks.

Mutex with Priority Inheritance

Protects the LCD display from concurrent access.

if (os_mut_wait(&mut_lcd, 100) == OS_R_OK) {

LCD_puts((U8*)line);

os_mut_release(&mut_lcd);

}

- Used In: display_task(), logger_task(), emergency_task()

- RTOS Feature: If a low-priority task holds the mutex, a higher-priority waiting task inherits its priority temporarily to prevent priority inversion.

Priority Ceiling – ICPP Simulation

Immediate Ceiling Priority Protocol simulated via icpp_acquire() and icpp_release() functions.

while (!icpp_acquire(self)) os_dly_wait(5);

update_temp_leds(level);

icpp_release(self);

- Used In: temp_task()

- Concept: When a task acquires a resource (like LEDs), no other task can preempt it until it releases the resource.

- Simulation: Uses global ceiling_owner variable.

| Variable | Role |
| --- | --- |
| ceiling_owner | Tracks which task holds the resource |
| mut_ceiling | Protects access to ceiling variables |

Priority Ceiling – OCPP Simulation

Original Ceiling Priority Protocol simulated via ocpp_acquire() / ocpp_release().

if (ocpp_acquire(self, LIGHT_RESOURCE_CEILING)) {

update_light_leds(level);

ocpp_release(self, LIGHT_RESOURCE_CEILING);

}

- Used In: light_task()

- Resource Ceiling: LIGHT_RESOURCE_CEILING = 2

- Concept:

  - Task can acquire resource only if its base priority ≤ ceiling.

  - Prevents low-priority tasks from blocking higher ones indirectly.

- Simulation Variables:

  - system_ceiling (global ceiling state)

  - task_table[] (records task base priorities)

Tables of Synchronization and Communication

Semaphore Usage

| Semaphore | Protected Resource | Used By Tasks | Timeout |
| --- | --- | --- | --- |
| sem_sensors | Shared sensors (temp, light, motion) | temp_task, light_task, display_task | 50 ticks |

Mutex Usage

| Mutex | Protected Resource | Tasks Using It | Purpose |
| --- | --- | --- | --- |
| mut_lcd | LCD display | display_task, logger_task, emergency_task | Avoid concurrent LCD access |
| mut_ceiling | ICPP/OCPP state | temp_task, light_task | Protects ceiling manager variables |

Message Box

| Message Box | Size | Producer Tasks | Consumer Task |
| --- | --- | --- | --- |
| msgbox | 5 | temp_task, light_task | logger_task |

Event Flags

| Flag | Meaning | Sender | Receiver |
| --- | --- | --- | --- |
| EVT_TEMP_UPDATE | Temperature update | temp_task | display_task |
| EVT_LIGHT_UPDATE | Light update | light_task | display_task |
| EVT_MOTION | Motion detected | motion_task | light_task, display_task |

System Behavior Summary

| Condition | Action Taken |
| --- | --- |
| Temperature rises above threshold | Fan LEDs increase gradually |
| Room becomes dark | Light LEDs increase |
| Motion detected | All LEDs flash (safety override) |
| Temperature > 45°C | Emergency task triggers LCD alert and flashing LEDs |
| System idle | Clock LED blinks periodically |

Example LCD Output

Smart Home System

T:30C L:45 M:1

Log:Temp:30C Fan:2

Possible Enhancements

- Add real sensor interface via ADC instead of simulation.

- Implement true priority ceiling protocol using actual os_tsk_prio() API calls.

- Add UART logging or SD card data storage.

- Introduce network control (e.g., MQTT or Wi-Fi).

Result Outputs:

Conclusion

This project effectively demonstrates multiple RTOS synchronization mechanisms in a cohesive smart home simulation.
It showcases:

- Controlled concurrency

- Safe inter-task communication

- Prevention of race conditions

- Handling of real-time constraints

It provides a practical educational platform for understanding how advanced scheduling protocols like ICPP and OCPP can be implemented and observed in embedded systems.